ME155A

Team #1

Bill of Materials

| Component | Stock Material | Description | Price |
| --- | --- | --- | --- |
| Rectangular Box x 1 | Boise ¼ in 2 ft. x 4 ft. MDF | The box comprises four panels that are to be machined out of the stock | $5.27 x 2 |
| Handle Adjust Block x 2 |  |  |  |
| Modified Handle Adjust x 1 |  |  |  |
| Handle x 1 | Delrin; McMaster-Carr Item#8572K61 | Requires lathe to machine | $4.45 |
| Handle Grip x 1 |  |  |  |
| 48 D.P., 8 Teeth, 14.5° Pressure. Angle, Brass Gear x 2 | SDP/SI: P/N# A 1B 1-N48008 | Gear for handle and for the fan | $5.38 x 2 |
| 48 D.P., 96 Teeth, 14.5° Pressure. Angle, Brass Gear x 3 | SDP/SI: P/N# A 1B 2-N48096 | Gear for main crank shaft | $29.04 x 3 |
| 48 D.P., 12 Teeth, 14.5° Pressure. Angle, Brass Gear x 4 | SDP/SI: P/N# A 1B 1-N48012 | Small connecting gear in transforming the rotational motion of the hand-crank into the motion of the fan | $5.87 x 4 |
| 18 Teeth, 48DP / Commercial Miter Gear x 2 | SDP/SI: P/N# A 1B 4-Y48018 | Translates rotational motion to allow the fan to turn | $8.83 x 2 |
| Shaft x 3 | Aluminum McMaster-Carr Item#1915T123 | Provides means for axial rotation | $15.97 |
| Pins x 5 |  | Pushes against the valves |  |
| Valve x 5 Valve Holder x 1 | Midwest Products Co. 7016 Balsa Block, 3 x 3 x 12 in. | Machined out of stock | $6.00 |
| Flute x 1 | Flexpvc: Sch 40; ½ in. x 1 ft. rigid pvc | Makes sound | $0.66 |
| Fan x 1 | Antec 3 x 3 in. Fan Unit | Provides a source for moving air | $2.00 |
| Funnel x 1 | Winco Plastic 16-Oz Funnel, 5-1/4” Diameter | Purchase from retailer, modify to fit box if necessary; provides air pressure | $0.72 |
| Rubber Disc x 5 | McMaster-Carr Item#6289K14 | Machined out of rubber stock, seals the holes in flute | $2.18 |
|  |  | Total | $181.54 |