C Notes

Computer Architecture:

ALU - basic low level logic processing

Data Memory - where numbers from computations are stored (temporarily) - RAM

Program Memory - holds actual list of commands

Quartz Clock - tells the ALU to go to the next line in code

32bit vs 64bits means that many bits can be populated in the data memory at once

Bus - connects all of these components

*all of these have limitations on speed, capacity, etc.

Pointer - address in data memory where a value is stored

Compiling C

1. C code translates to assembly

2. assembly assembled into machine code

3. machine code linked into an executable

4. linker translates functions in c into assembly/machine code, so now the ALU knows exactly what to execute

#include <stdio.h> -> (standard input and output library)

strings: like an array of numbers, ends with a \0 (null) - ending piece of string.

"hello" -> 'h' 'e' 'l' 'l' 'o' '\0'

data storage

char - 1 byte (signed or unsigned)

int - 2-4 bytes (signed or unsigned)

float - bigger, keeps track of decimal

double - double the size of a float (in memory size)

main() - default function that gets run first

void main(void) - will not take in or return anything