import csv

print "Welcome."

print "You are about to generate G-Code for a Dynamic Test"

print " "

oscillations=input("How many oscillations would you like to do? (Input a number) ") #number of oscillations you want the machine to do

frequency=input("What is the frequency of your oscillations? (Input a number in Hz) ") #how fast it goes

amplitude=input("What is the amplitude of each oscillation? (Input a number in mm) ") #how large of a sine wave you want to create

samplenum=input("Number of increments per cycle? ") #number of steps to create the sine wave function.

filename=raw_input("What is your file name? (Input a string) ")

date=raw_input("What is today's date? (Input a string) ")

feedrate=4*amplitude*frequency*60

sinwavevalues=['0'] #creates an array that holds the location values

coswavevalues=[str(feedrate)] #creates an array that holds the feedrate values

fileoutput=['N100 G01 G54 G90 G21'] #fileoutput is an array that contains the G code.

#N*** indicates line number, G54 is the set of coordinates, G01 creates linear tool paths,

#G90 makes the movement absolute (meaning based around 0), and G21 means mm

#the G code format is Nxxx Z**.**** F&&.&&&&, where xxx is the step number, Z is the axis, **.**** is decimal indicating the location

#of the machining tool, F indicates feed rate, &&.&&&& is the desired feedrate

startlocation=0 #the program should find the Z coordinate of the CNC machine here if we find a way to actively communicate with the CNC

for i in range(samplenum*oscillations):

#adds a location string to the sinwavevalue array and the feedrate string to the coswavevalue array

sinwavevalues.append(str(round((startlocation+amplitude*math.sin(2*math.pi*(float(i+1)/float(samplenum))*frequency)),4)))

coswavevalues.append(str(abs(round((feedrate*math.cos(2*math.pi*(float(i+1)/float(samplenum))*frequency)),4))))

#checks for any 0 feedrates and replaces them with the previous value for the feedrate

if (float(coswavevalues[i])<0.000001):

coswavevalues[i]=coswavevalues[i-1]

#creates the formatting for the file output array

fileoutput.append('N'+str(101+i)+' Z'+sinwavevalues[i]+' F'+coswavevalues[i])

#Here begins the data saving part

savefilename=(filename+'.nc')

savestring=[["("+date+")"],["(Dynamic Test)"],["(Oscillations: "+str(oscillations)+")"],["(Frequency: "+str(frequency)+"Hz)"],["(Amplitude: "+str(amplitude)+"mm)"],["(Maximum Feed Rate: "+str(feedrate)+"mm/min)"],[" "]]

for d in fileoutput:

savestring +=[[d]]

datafileobj = open(savefilename, 'wb')

datafile = csv.writer(datafileobj)

datafile.writerows(savestring)

datafileobj.close()

print " "

print "The G-Code has been created"